INTRODUCCIÓN

© Alchemist 1997.

This is a compilation of information gathered over several years based on a more or less in-depth study of the CASIO FX-850P calculator. Some chapters have been generously provided by third parties. Others have been completed to a greater or lesser extent. Thanks to all of them.

There may still be many more hidden functions of the calculator. This guide has attempted to cover most of them (perhaps all). However, studying this guide may open new horizons in understanding the calculator's structure.

The reader will experience greater control over the calculator, allowing them to use it according to their convenience.

Caution: When accessing many of these new possibilities, such as access to system memory, system pointers, etc., special care must be taken, as there is a risk of altering the system.

It is advisable to use the communications interface and transfer all critical data to the PC. The communications interface is not described here.

DIRECCIONAMIENTO DE LA MEMORIA RAM

It is possible to address RAM memory from a range of -32769 to 65536 (both addresses are exclusive).

The negative addresses are added to 65536, this new address being the final one.

Example: direction -1 (-1+65536) 65535

address -32768 (-32768+65536) 32767

The calculator's RAM memory is cyclic and, like PC memory, has various addressing methods.

If we want to address a byte in segment 0 and position 1024, this address will be expressed as follows: 0:1024.

However, this is not the only way to represent it, although the same byte could also be addressed by the segment:offset pair, 1:1008 or 2:992, etc. In any case, we always refer to byte 1024 of segment 0 (taking this segment as absolute), or in other words, to the actual address.

The calculation of the actual direction is as follows:

16 x Segment number + displacement

Example: direction 2: 256

When the byte is read at the previous address, the byte at address 256 is not read, but the byte at address (16*2+256=288).

Depending on the type of RAM memory expansion (RP-8 (8 Kb)), (RP-33 (32 Kb)) the total memory can be 16 Kb or 40 Kb.

In the case of 40 Kb memory, the end of the RAM is at address 40960. From this address onwards, the memory becomes cyclic, that is, the byte 40960+1 will be like addressing the beginning of the memory (0:0).

In summary, byte 40960 (in the case of having the maximum RAM expansion) will be the end of the capacity to store bytes, but not the end of the capacity to address beyond this limit.

The following figure shows a graphical representation of the calculator's memory.

TABLA DE PUNTEROS

The system stores essential information for managing programs, variables, special memory areas, and pointers.

This information is of very special value to both the user and the system.

Knowing how to use the pointers allows you to take almost total control of the calculator, as well as achieve some interesting effects.

| Address |  | Description |
| --- | --- | --- |
| 0: | 0000 | Screen buffer. |
| 0: | 0512 | Command input buffer, 0. |
| 0: | 0768 | BUFFER VALF( ) ( 155 bytes ),0. |
| 0: | 0512 | Command input buffer, 0. |
| 0: | 1024 | Formula storage mode (Function Memory),0. |
| 0: | 1610 | Program pointer where the last PB command was executed (PB-100 compatibility). |
| 0: | 1613 | SEG+DIR of READ#. |
| 0: | 1616 | Pointer of the pointer of the last program that was in BASIC. |
| 0: | 1619 | Pointer parameters for INPUT (Text if available, otherwise variable). |
| 0: | 1627 | After editing, if it is 13, EXE has been pressed; if it is 253, shift +EXE has been pressed. |
| 0: | 1628 | =0 LIB off; >0 LIB on. |
| 0: | 1629 | Angular mode; 0=DEG; 1 =RAD; 2=GRA. |
| 0: | 1630 | If it is equal to 0 DEFM off; >0 DEFM on. |
| 0: | 1673 | Start pointer of the character stack. |
| 0: | 1676 | Row pointer of the character stack. |
| 0: | 1682 | Stack head pointer FOR… NEXT. |
| 0: | 1685 | GOSUB Stack Head Pointer. |
| 0: | 1688 | GOSUB Stack Tail Pointer. |
| 0: | 1691 | End pointer of the system area. |
| 0: | 1697 | Starting pointer of the variable table. |
| 0: | 1700 | Pointer to the last byte of variables. |
| 0: | 1703 | Pointer to the first byte of MEMO. |
| 0: | 1706 | Pointer to last byte of MEMO. |
| 0: | 1709 | MEMO pointer. |
| 0: | 1712 | Pointer of the pointer of P9 (Pointer table). |
| 0: | 1715 | Pointer of the pointer of P0 (Pointer table). |
| 0: | 1718 | Library pointer address 5:63406. |
| 0: | 1737 | =2 PRT on. |
| 0: | 1750 | Pointer to the last executed program command. |
| 0: | 1753 | DATA Pointer |
| 0: | 1759 | Pointer ON ERROR GOTO |
| 0: | 1762 | =ERL ( 2 bytes ). |
| 0: | 1767 | =ERR ( 1 byte ). |
| 0: | 1774 | Running program line, last edited or entered (2 bytes). |
| 0: | 1776 | Pointer to the command that is being executed. |
| 0: | 1785 | ANS (as variables, 8 bytes). |
| 0: | 1820 | =4 CAPS on, =0 lowercase (read only). |
| 0: | 1858 | Pointer to the pointer of the last executed library program. |
| 0: | 1867 | Buffer PASS ( 8 bytes ASCIIZ = terminated in character 0 ). |
| 0: | 40959 | End of RAM |
| 5: | 63406 | Table of pointers for library programs. |

ALMACENAMIENTO DE VARIABLES Y PUNTEROS

This chapter will show how program, library, and data bank (MEMO) pointer tables, variables, pointers, and pointer tables are stored in RAM.

The beginning of the variable table is given by the start pointer of the variable table at address [0:1697] (see chapter Pointer Table ). Before describing the structure of the variable table, we will learn how to obtain the pointers, as well as their structure.

The structure of a pointer is as follows: (each frame is a byte)

BL

MSB

LSB

LSB = Least significant byte of the pointer

MSB = Most significant byte of the pointer

BL = Memory Block Segment

The formula to obtain the memory address that the pointer points to is as follows:

P=PEEK(LSB)+256*PEEK(MSB)

The variable P will be the desired memory address.

The Block Segment will be used with the BASIC DEFSEG instruction.

DEFSEG=BL*4096

In this way we will have a memory address with segment: offset.

Segment 0 corresponds to RAM and segments 4 and 5 to ROM.

Returning to the structure of the variables, we will calculate the beginning of the variable table using the procedure described above:

PI=PEEK1697+256*PEE1698

The starting point PI is the memory address where the first variable will be located.

Using also the pointer, the last byte of the variable table, from the pointer table we will have the address where the last variable ends.

PF=PEEK1700+256*PEE1701

The segments have been omitted because we know that the variables are in RAM memory (DEFSEG=0).

PI 								PF

Variable

Variable

Variable

Variable

Cada estructura de variable tiene un tamaño distinto según el nombre de la variable y el tipo de variable.

Dirección de la variable

Nombre de la variable

Longitud del nombre de la variable*16+Tipo

Tipo	= 0 si la variable es una string

=4 si es un Número

=8 si es una matriz de strings

=12 si es una matriz de números

Se sumará 2 si el valor es tecleado y nada si es un valor calculado (más precisión)

The value of the variable will begin at this address. The length of the value will depend on whether it is a number or a string.

This address is in reverse notation. A practical example of a variable table is the following listing of RAM memory.

There are two numeric variables S and CH2 and one string variable CHAR$.

| Length name | Name | Address | Hexadecimal/ ASCII |
| --- | --- | --- | --- |
| 14 (1*16+4=20=&H14) | 53 CHR$(53)="S" | 6E 27 &H276E | 14 53 6E 27 .Sn' |
| 34 (3*16+4=52=&H34) | 43 48 32 “C”+”H”+”2” | 76 27 &H2776 | 34 43 48 32 76 27 4CH2v' |
| 40 (4*16+0)=64=&H40 | 43 48 41 52 “C”+”H”+”A”+”R” | C0 0C &H0CC0 | 40 43 48 41 52 C0 0C @CHARÀ. |

Note that the last variable is of type string (CHAR$) and the character “$” does not appear in memory.

The bytes in hexadecimal appear consecutively:

14 53 6E 27 34 43 48 32 76 27 40 43 48 41 52 C0 0C

Looking in the direction of the variable we can have numbers or text strings:

1 BYTE

Number: 	A.BCDEFGHIJKL*10 xy

MN

LT

JK

HI

FG

DE

BC

0A

The exponent depends on whether the number is greater than or equal to 1 and/or less than zero. The variable T controls this dependency.

T=(1 if the number is >=1) + (5 if the number is <0)

MN is calculated with respect to this parameter.

The table could look like this:

| T | MN |
| --- | --- |
| 0 | 100-ABS(XY) |
| 5 | 100-ABS(XY) |
| 1 | ABS(XY) |
| 6 | ABS(XY) |

Example 1:

Variable XX=-0.23456789012

In memory, a conversion to scientific notation is performed:

conversion: -2.3456789012E-1

Memory:

99

05

12

90

78

56

34

02

Example 2:

Variable XX=-0.23456789012E-18

In memory, a conversion to scientific notation is performed:

conversion: -2.3456789012E-19

Memory:

81

05

12

90

78

56

34

02

Example 3:

Variable XX=256789012E-18

In memory, a conversion to scientific notation is performed:

conversion: 2.56789012E-10

Memory:

90

00

00

12

90

78

56

02

Note: All numbers are in hexadecimal 	.

Alphanumeric string (text): “example of a text string”

This type of variable is much simpler than the previous one. It consists only of the length of the text string plus its ASCII equivalent.

Longitud

Letras ASCII

LISTADO DE LA MEMORIA ROM

The ROM memory starts in block segment 4.

To access this memory address, you must use the instruction DEFSEG=4*4096.

The program library (libraries) begins in the area listed here.

This section shows the beginning of the ROM. Some important areas of the ROM are shown in different colors.

Puntero de códigos de teclado

Inicio de la ROM

códigos de teclado

Inicio de tabla de punteros de librería

Dirección de inicio librería

Final de tabla de punteros de librería

Dirección final de librería

| Director | Hexadecimal | ASCII |
| --- | --- | --- |
| 0000 | 43 4D 04 FF FF 01 56 0B 1B 00 57 55 30 30 30 31 | CM....V...WU0001 |
| 0010 | 00 00 04 40 00 04 60 01 04 B0 01 04 FF FF FF FF | ...@..`..°...... |
| 0020 | FF FF FF FF FF FF FF FF FF FF FF FF FF FF 90 05 | .............. . |
| 0030 | 04 24 E0 05 24 E0 05 BD F7 05 BD F7 05 00 00 06 | .$à.$à.½÷.½÷.... |
| 0040 | 00 00 FE 00 DC DB DA 03 2F 5E 8A F0 FA DE DD 00 | ....ÜÛÚ./^ŠðúÞÝ. |
| 0050 | 2D 2A 08 A1 89 F5 DF 00 0D 2B 36 39 29 88 F4 00 | -*..¡‰õß..+69)ˆô. |
| 0060 | AD 33 35 38 28 87 DA FC 2E 30 32 31 34 37 85 81 | 358(‡Ú..02147… |
| 0070 | 00 20 00 F6 F2 0E AE 80 2C 3A 3D 4C 50 4F 12 1C | . .öò.®€ :=LPO.. |
| 0080 | 3B 4D 4B 4A 49 55 1D 1F 4E 42 48 47 59 54 1E FB | ;MKJIU..NBHGYT.û |
| 0090 | 56 43 46 44 52 45 0F 0F 58 5A 53 41 57 51 FF 00 | VCFDRE..XZSAWQ.. |
| 00A0 | 00 00 FE 00 DC DB DA 03 A8 A3 8D F0 FA DE DD 00 | ....ÜÛÚ.¨£ ðúÞÝ. |
| 00B0 | A5 A7 0C 9E 8C F5 DF 00 FD A6 D6 D9 A2 8B F4 00 | ¥§.žŒõß..¦ÖÙ¢‹ô. |
| 00C0 | 86 D3 D5 D8 A4 9D DA FC 9C D0 D2 D1 D4 D7 84 82 | †ÓÕØ¤ Ú.œÐÒÑÔ×„‚ |
| 00D0 | 00 20 00 13 F3 04 83 AF 3F 3E 40 97 5C 5D 11 06 | . ..ó.ƒ¯?>@—\].. |
| 00E0 | 3C 93 98 96 5B 27 02 1F 9F A0 95 94 26 25 0B FB | <“˜–['..Ÿ •”&%.û |
| 00F0 | 9B 99 92 91 24 23 0F 1A 9A 8F 90 8E 22 21 FF 00 | ›™''$#..š Ž"!.. |
| 0100 | 15 78 15 77 15 79 19 A3 19 97 18 9C 15 60 15 AC | .xwy£.—.œ.`.¬ |
| 0110 | 15 6B 15 6C 15 6D 15 6E 15 6F 15 70 14 4A 14 A3 | .klmnopJ£ |
| 0120 | 14 4B 14 49 14 81 14 6D 14 82 14 8D 17 47 14 57 | .KI .m.‚. .GW |
| 0130 | 14 6F 14 6A 16 9B 14 7C 15 93 14 AC 15 63 14 6C | .oj›.¦.“.¬.cl |
| 0140 | 14 59 15 7A 15 7F 15 67 15 7D 18 A7 18 A8 18 AA | .Yz .g.}.§.¨.ª |
| 0150 | 18 AB 00 00 00 00 00 00 00 00 7F CC 7F CE 7F D2 | .«........ Ì Î Ò |
| 0160 | 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 | ................ |
| 0170 | 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 | ................ |
| 0180 | B6 FE 05 B6 FE 05 B6 FE 05 B6 FE 05 B6 FE 05 BD | ¶..¶..¶..¶..¶..¶..½ |
| 0190 | F7 05 DB F7 05 DB F7 05 FF FF FF FF FF FF FF FF | ÷.Û÷.Û÷......... |
| 01A0 | FF FF FF FF FF FF FF FF FF FF FF FF FF F9 F7 05 | .............ù÷. |
| 01B0 | B2 03 B3 03 B4 03 B5 03 B6 03 B7 03 B8 03 B9 03 | ².³.´.µ.¶.·.¸.¹. |
| 01C0 | BA 03 BB 03 BC 03 BD 03 BE 03 BF 03 C0 03 C1 03 | º.».¼.½.¾.¿.À.Á. |
| 01D0 | C2 03 C3 03 C4 03 C5 03 C6 03 C7 03 C8 03 C9 03 | Â.Ã.Ä.Å.Æ.Ç.È.É. |
| 01E0 | CA 03 CB 03 CC 03 CD 03 CE 03 CF 03 D0 03 D2 03 | Ê.Ë.Ì.Í.Î.Ï.Ð.Ò. |
| 01F0 | D3 03 D4 03 D5 03 D6 03 D7 03 D8 03 D9 03 DA 03 | Ó.Ô.Õ.Ö.×.Ø.Ù.Ú. |
| 0200 | DB 03 DC 03 DD 03 DE 03 DF 03 E0 03 E1 03 E2 03 | Û.Ü.Ý.Þ.ß.à.á.â. |
| 0210 | E3 03 E4 03 E5 03 E6 03 E7 03 E8 03 E9 03 EA 03 | ã.ä.å.æ.ç.è.é.ê. |
| 0220 | EB 03 EC 03 ED 03 ED 03 ED 03 EE 03 EE 03 EF 03 | ë.ì.í.í.í.î.î.ï. |
| 0230 | EF 03 F0 03 F1 03 F2 03 F3 03 F4 03 F5 03 F6 03 | ï.ð.ñ.ò.ó.ô.õ.ö. |
| 0240 | F7 03 F8 03 F9 03 FA 03 FB 03 FC 03 FD 03 FE 03 | ÷.ø.ù.ú.û....... |
| 0250 | FF 03 00 04 01 04 02 04 03 04 04 04 05 04 06 04 | ................ |
| 0260 | 07 04 08 04 09 04 09 04 09 04 0A 04 0B 04 0C 04 | ................ |
| 0270 | 0D 04 0E 04 0F 04 10 04 11 04 12 04 13 04 14 04 | ................ |
| 0280 | 15 04 16 04 17 04 18 04 19 04 1A 04 1B 04 1C 04 | ................ |
| 0290 | 1D 04 1E 04 1F 04 20 04 21 04 22 04 23 04 24 04 | ...... .!.".#.$. |
| 02A0 | 25 04 26 04 27 04 28 04 29 04 29 04 29 04 29 04 | %.&.'.(.).).).). |
| 02B0 | 29 04 2D 04 31 04 35 04 39 04 3D 04 41 04 45 04 | ).-.1.5.9.=.AE |
| 02C0 | 48 04 4C 04 50 04 54 04 59 04 5D 04 61 04 67 04 | HLPTY].ag |
| 02D0 | 6B 04 6F 04 73 04 7A 04 81 04 85 04 89 04 8F 04 | kosz .….‰. . |
| 02E0 | 95 04 99 04 9E 04 A3 04 A8 04 AF 04 B2 04 B7 04 | •.™.ž.£.¨.¯.².·. |
| 02F0 | BB 04 BF 04 C4 04 CA 04 D1 04 D8 04 DB 04 E1 04 | ».¿.Ä.Ê.Ñ.Ø.Û.á. |
| 0300 | E7 04 EC 04 F1 04 F5 04 F8 04 FD 04 05 05 0A 05 | ç.ì.ñ.õ.ø....... |
| 0310 | 0E 05 14 05 18 05 1C 05 20 05 26 05 2B 05 2D 05 | ........ .&.+.-. |
| 0320 | 31 05 35 05 38 05 3C 05 41 05 45 05 49 05 4D 05 | 1.5.8.<.AEIM |
| 0330 | 51 05 56 05 5A 05 5E 05 64 05 68 05 6B 05 6E 05 | QVZ^.dhkn |
| 0340 | 72 05 76 05 7A 05 81 05 85 05 85 05 85 05 85 05 | rvz .….….….…. |
| 0350 | 85 05 89 05 8F 05 8F 05 8F 05 8F 05 8F 05 8F 05 | ….‰. . . . . . . |
| 0360 | 8F 05 8F 05 8F 05 8F 05 8F 05 8F 05 8F 05 8F 05 | . . . . . . . . |
| 0370 | 8F 05 8F 05 8F 05 8F 05 8F 05 8F 05 8F 05 8F 05 | . . . . . . . . |
| 0380 | 8F 05 8F 05 8F 05 8F 05 8F 05 8F 05 8F 05 8F 05 | . . . . . . . . |
| 0390 | 8F 05 8F 05 8F 05 8F 05 8F 05 8F 05 8F 05 8F 05 | . . . . . . . . |
| 03A0 | 8F 05 8F 05 8F 05 8F 05 8F 05 8F 05 8F 05 90 05 | . . . . . . . . |
| 03B0 | 90 05 20 2B 2D 2A 2F E3 21 22 23 24 3E E0 3D E1 | . +-*/ã!"#$>à=á |
| 03C0 | 3C E2 30 31 32 33 34 35 36 37 38 39 2E E7 29 28 | <â0123456789.ç)( |
| 03D0 | 45 2D 45 41 42 43 44 45 46 47 48 49 4A 4B 4C 4D | E-EABCDEFGHIJKLM |
| 03E0 | 4E 4F 50 51 52 53 54 55 56 57 58 59 5A 8A 8E 61 | NOPQRSTUVWXYZŠŽa |
| 03F0 | 62 63 64 65 66 67 68 69 6A 6B 6C 6D 6E 6F 70 71 | bcdefghijklmnopq |
| 0400 | 72 73 74 75 76 77 78 79 7A 3F 2C 3B 3A ED 84 DF | rstuvwxyz?;:í„ß |
| 0410 | EE 40 F0 9F E8 E4 E9 EA EB 8D 85 E5 E6 25 5C EC | î@ðŸèäéêë …åæ%\ì |
| 0420 | 5B 26 5F 27 A5 5D 87 EF 86 53 49 4E 20 43 4F 53 | [&_'¥]‡ï†WITHOUT COS |
| 0430 | 20 54 41 4E 20 41 53 4E 20 41 43 53 20 41 54 4E | TAN ASN ACS ATN |
| 0440 | 20 4C 4F 47 20 4C 4E 20 45 58 50 20 53 51 52 20 | LOG LN EXP SQR |
| 0450 | 49 4E 54 20 46 52 41 43 20 41 42 53 20 53 47 4E | INT FRAC ABS SGN |
| 0460 | 20 52 4F 55 4E 44 28 52 41 4E 23 4C 45 4E 28 56 | ROUND(RAN#LEN(V |
| 0470 | 41 4C 28 4D 49 44 24 28 24 2C 49 4E 4B 45 59 24 | AL(MID$($ INKEY$ |
| 0480 | 20 43 53 52 20 20 54 4F 20 20 53 54 45 50 20 20 | CSR TO STEP |
| 0490 | 54 48 45 4E 20 44 45 47 28 48 45 58 24 28 53 54 | THEN DEG(HEX$(ST |
| 04A0 | 52 24 28 44 4D 53 24 28 4D 49 44 24 28 24 2C 41 | R$(DMS$(MID$($A |
| 04B0 | 4C 4C 42 45 45 50 20 4C 45 54 20 46 4F 52 20 4E | LLBEEP LET FOR N |
| 04C0 | 45 58 54 20 20 47 4F 54 4F 20 20 47 4F 53 55 42 | EXT GOTO GOSUB |
| 04D0 | 20 52 45 54 55 52 4E 20 49 46 20 50 52 49 4E 54 | RETURN IF PRINT |
| 04E0 | 20 49 4E 50 55 54 20 4D 4F 44 45 20 53 54 4F 50 | INPUT MODE STOP |
| 04F0 | 20 45 4E 44 20 4F 4E 20 52 45 41 44 20 52 45 53 | END ON READ RES |
| 0500 | 54 4F 52 45 20 44 41 54 41 20 52 45 4D 20 43 4C | TOREP DATA REM CL |
| 0510 | 45 41 52 20 53 45 54 20 3F 3F 3F 20 3F 3F 3F 20 | EAR SET ??? ??? |
| 0520 | 43 4C 45 41 52 20 44 45 46 4D 20 26 48 43 55 52 | CLEAR DEFM &HCUR |
| 0530 | 20 50 4F 4C 28 48 59 50 52 45 43 28 46 41 43 54 | POL(HYPREC(FACT |
| 0540 | 20 4E 50 52 28 3F 3F 3F 20 4E 43 52 28 3F 3F 3F | NPR(??? NCR(??? |
| 0550 | 20 44 45 46 4D 20 53 41 56 45 4C 4F 41 44 56 45 | DEFM SAVELOADVE |
| 0560 | 52 49 46 59 4C 49 53 54 52 55 4E 4E 45 57 50 41 | RIFYLISTRUNNEWPA |
| 0570 | 53 53 3F 3F 3F 20 3F 3F 3F 20 57 52 49 54 45 23 | SS??? ??? WRITE# |
| 0580 | 20 3F 3F 3F 20 44 49 4D 20 45 52 41 53 45 20 3rd | ??? DIM ERASE : |

LOCALIZACIÓN  DE LA BIBLIOTECA (LIBRERIAS)

The program library consists of 116 different programs according to the user manual.

However, there are several undocumented programs and routines that are called from other libraries. This makes a total of 141 different programs.

These programs are located in ROM memory modules, occupying up to 128 Kb (2 modules of 64 Kb).

To obtain its exact location, it is necessary to obtain the necessary information from the library pointer table (Start address, end address, Library name…)

The library pointer table is composed of groups of fifteen bytes, one for each library. This table is located in segment 5 of ROM memory.

The sample localization program could be the following:

| 10 20 30 40 50 60 70 | DEFSEG=5*4096 ; Address segment 5 NL=1:D=NL*15 ; NL= Library number P=63406+D ; Position of the pointer table START=PEEKP+256*PEEK(P+1 ; Get the start of the library FIN=PEEK(P+3)+256*PEEK(P+4 ; Get the end of the library NUM$=CHR$(PEEK(P+7))+CHR$(PEEK(P+8))+CHR$(PEEK(P+9))+ CHR$(PEEK(P+10))+ CHR$(PEEK(P+11)) PRINT NUM$ ;Prints library number (e.g., 5500) |
| --- | --- |

The following table includes the location of all library programs, as well as their length.

This data has been transmitted from the calculator to the PC computer using the interface of this project, using a program similar to the one shown on the previous sheet.

The first number before the colon in the Start and End values is the ROM block or segment where the library is located.

| N. Bookstore | Start | End | Length |
| --- | --- | --- | --- |
| 0400 | 4:01424 | 4:03285 | 1861 bytes |
| 0800 | 4:03286 | 4:04415 | 1129 bytes |
| 0900 | 4:04416 | 4:08267 | 3851 bytes |
| 1000 | 4:08268 | 4:09474 | 1206 bytes |
| 5010 | 4:09475 | 4:10180 | 0705 bytes |
| 5020 | 4:10181 | 4:10631 | 0450 bytes |
| 5040 | 4:10632 | 4:12286 | 1654 bytes |
| 5050 | 4:12287 | 4:13496 | 1209 bytes |
| 5060 | 4:13497 | 4:15486 | 1989 bytes |
| 5080 | 4:15487 | 4:16594 | 1107 bytes |
| 5090 | 4:16595 | 4:17769 | 1174 bytes |
| 5100 | 4:17770 | 4:23005 | 5235 bytes |
| 5200 | 4:23006 | 4:24275 | 1269 bytes |
| 5220 | 4:24276 | 4:25520 | 1244 bytes |
| 5230 | 4:25521 | 4:26377 | 0856 bytes |
| 5250 | 4:26378 | 4:27098 | 0720 bytes |
| 5260 | 4:27099 | 4:27951 | 0852 bytes |
| 5270 | 4:27952 | 4:28945 | 0993 bytes |
| 5280 | 4:28946 | 4:29790 | 0844 bytes |
| 5290 | 4:29791 | 4:31029 | 1238 bytes |
| 5300 | 4:31030 | 4:33196 | 2166 bytes |
| 5350 | 4:33197 | 4:36466 | 3269 bytes |
| 5510 | 4:36467 | 4:37092 | 0625 bytes |
| 5520 | 4:37093 | 4:37484 | 0391 bytes |
| 5530 | 4:37485 | 4:37848 | 0363 bytes |
| 5540 | 4:37849 | 4:38358 | 0509 bytes |
| 5550 | 4:38359 | 4:39153 | 0794 bytes |
| 5560 | 4:39154 | 4:39587 | 0433 bytes |
| 5570 | 4:39588 | 4:41303 | 1715 bytes |
| 5600 | 4:41304 | 4:42141 | 0837 bytes |
| 5605 | 4:42142 | 4:42512 | 0370 bytes |
| 5610 | 4:42513 | 4:43123 | 0610 bytes |
| 5615 | 4:43124 | 4:43386 | 0262 bytes |
| 5620 | 4:43387 | 4:43984 | 0597 bytes |
| 5625 | 4:43985 | 4:44404 | 0419 bytes |
| 5630 | 4:44405 | 4:44736 | 0331 bytes |
| 5635 | 4:44737 | 4:45444 | 0707 bytes |
| 5650 | 4:45445 | 4:45714 | 0269 bytes |
| 5655 | 4:45715 | 4:46162 | 0447 bytes |
| 5660 | 4:46163 | 4:46601 | 0438 bytes |
| 5665 | 4:46602 | 4:46949 | 0347 bytes |
| 5670 | 4:46950 | 4:47337 | 0387 bytes |
| 5675 | 4:47338 | 4:47735 | 0397 bytes |
| 5700 | 4:47736 | 4:48013 | 0277 bytes |
| 5705 | 4:48014 | 4:48443 | 0429 bytes |
| 5710 | 4:48444 | 4:48789 | 0345 bytes |
| 5715 | 4:48790 | 4:49130 | 0340 bytes |
| 5720 | 4:49131 | 4:49475 | 0344 bytes |
| 5725 | 4:49476 | 4:49899 | 0423 bytes |
| 5730 | 4:49900 | 4:50305 | 0405 bytes |
| 5735 | 4:50306 | 4:50674 | 0368 bytes |
| 5740 | 4:50675 | 4:51165 | 0490 bytes |
| 5745 | 4:51166 | 4:51536 | 0370 bytes |
| 5750 | 4:51537 | 4:52187 | 0650 bytes |
| 5760 | 4:52188 | 4:53604 | 1416 bytes |
| 5800 | 4:53605 | 4:54970 | 1365 bytes |
| 5810 | 4:54971 | 4:56758 | 1787 bytes |
| 5820 | 4:56759 | 4:58361 | 1602 bytes |
| 5830 | 4:58362 | 4:59957 | 1595 bytes |
| 5840 | 4:59958 | 4:61807 | 1849 bytes |
| 5900 | 4:61808 | 5:1165 | 4892 bytes |
| 5910 | 5:1166 | 5:3487 | 2321 bytes |
| 5920 | 5:3488 | 5:4374 | 0886 bytes |
| 5930 | 5:4375 | 5:5800 | 1425 bytes |
| 5932 | 5:5801 | 5:7002 | 1201 bytes |
| 5934 | 5:7003 | 5:8199 | 1196 bytes |
| 5936 | 5:8200 | 5:9507 | 1307 bytes |
| 5938 | 5:9508 | 5:10560 | 1052 bytes |
| 5950 | 5:10561 | 5:12473 | 1912 bytes |
| 5960 | 5:12474 | 5:13497 | 1023 bytes |
| 5970 | 5:13498 | 5:16382 | 2884 bytes |
| 5980 | 5:16383 | 5:17331 | 0948 bytes |
| 6210 | 5:17332 | 5:17676 | 0344 bytes |
| 6220 | 5:17677 | 5:18128 | 0451 bytes |
| 6230 | 5:18129 | 5:18577 | 0448 bytes |
| 6240 | 5:18578 | 5:19129 | 0551 bytes |
| 6310 | 5:19130 | 5:19632 | 0502 bytes |
| 6320 | 5:19633 | 5:20094 | 0461 bytes |
| 6330 | 5:20095 | 5:20641 | 0546 bytes |
| 6410 | 5:20642 | 5:21006 | 0364 bytes |
| 6420 | 5:21007 | 5:21476 | 0469 bytes |
| 6430 | 5:21477 | 5:21945 | 0468 bytes |
| 6440 | 5:21946 | 5:22515 | 0569 bytes |
| 6450 | 5:22516 | 5:22680 | 0164 bytes |
| 6460 | 5:22681 | 5:22879 | 0198 bytes |
| 6500 | 5:22880 | 5:24426 | 1546 bytes |
| 6510 | 5:24427 | 5:24451 | 0024 bytes |
| 6520 | 5:24452 | 5:24476 | 0024 bytes |
| 6530 | 5:24477 | 5:24501 | 0024 bytes |
| 6540 | 5:24502 | 5:24526 | 0024 bytes |
| 6610 | 5:24527 | 5:25309 | 0782 bytes |
| 6620 | 5:25310 | 5:26099 | 0789 bytes |
| 6630 | 5:26100 | 5:26860 | 0760 bytes |
| 6640 | 5:26861 | 5:27627 | 0766 bytes |
| 6650 | 5:27628 | 5:28697 | 1069 bytes |
| 6660 | 5:28698 | 5:29844 | 1146 bytes |
| 6670 | 5:29845 | 5:30457 | 0612 bytes |
| 6680 | 5:30458 | 5:31229 | 0771 bytes |
| 6710 | 5:31230 | 5:31256 | 0026 bytes |
| 6711 | 5:31257 | 5:31282 | 0025 bytes |
| 6712 | 5:31283 | 5:31308 | 0025 bytes |
| 6720 | 5:31309 | 5:31335 | 0026 bytes |
| 6721 | 5:31336 | 5:31361 | 0025 bytes |
| 6722 | 5:31362 | 5:31387 | 0025 bytes |
| 6730 | 5:31388 | 5:31414 | 0026 bytes |
| 6731 | 5:31415 | 5:31440 | 0025 bytes |
| 6732 | 5:31441 | 5:31466 | 0025 bytes |
| 6740 | 5:31467 | 5:31493 | 0026 bytes |
| 6741 | 5:31494 | 5:31519 | 0025 bytes |
| 6742 | 5:31520 | 5:31545 | 0025 bytes |
| 6750 | 5:31546 | 5:31572 | 0026 bytes |
| 6751 | 5:31573 | 5:31598 | 0025 bytes |
| 6752 | 5:31599 | 5:31624 | 0025 bytes |
| 6760 | 5:31625 | 5:31651 | 0026 bytes |
| 6761 | 5:31652 | 5:31677 | 0025 bytes |
| 6762 | 5:31678 | 5:31703 | 0025 bytes |
| 6770 | 5:31704 | 5:31730 | 0026 bytes |
| 6771 | 5:31731 | 5:31756 | 0025 bytes |
| 6772 | 5:31757 | 5:31782 | 0025 bytes |
| K0630 | 5:31783 | 5:39375 | 7592 bytes |
| M6510 | 5:39376 | 5:43178 | 3802 bytes |
| M6710 | 5:43179 | 5:44181 | 1002 bytes |
| M6720 | 5:44182 | 5:45199 | 1017 bytes |
| M6730 | 5:45200 | 5:46339 | 1139 bytes |
| M6740 | 5:46340 | 5:47587 | 1247 bytes |
| M6750 | 5:47588 | 5:48886 | 1298 bytes |
| M6760 | 5:48887 | 5:49733 | 0846 bytes |
| M6770 | 5:49734 | 5:50665 | 0931 bytes |
| S6020 | 5:50666 | 5:51949 | 1283 bytes |
| S6030 | 5:51950 | 5:53238 | 1288 bytes |
| S6210 | 5:53239 | 5:53551 | 0312 bytes |
| S6220 | 5:53552 | 5:53899 | 0347 bytes |
| S6230 | 5:53900 | 5:54135 | 0235 bytes |
| S6240 | 5:54136 | 5:54701 | 0565 bytes |
| S6310 | 5:54702 | 5:54890 | 0188 bytes |
| S6320 | 5:54891 | 5:55007 | 0116 bytes |
| S6330 | 5:55008 | 5:55311 | 0303 bytes |
| S6410 | 5:55312 | 5:55660 | 0348 bytes |
| S6420 | 5:55661 | 5:56153 | 0492 bytes |
| S6430 | 5:56154 | 5:56675 | 0521 bytes |
| S6440 | 5:56676 | 5:57379 | 0703 bytes |

LIBRERÍAS NO DOCUMENTADAS

There are several undocumented library programs. These programs correspond to the following numbers:

0400 -> Calculator testing program

0800 -> Printer program

0900 -> PB100 to FX-850P converter

To access the programs, simply (from MODE 0) enter the number and press the LIB key.

It is also possible to access the menu library by its own program number.

K0630 -> Library Menu

The following lists correspond to undocumented libraries:

Library: 0400 		Calculator Test

| 10 | CLEAR:CLS:DEFSEG=0:K=PEEK(&H6B4)/4+PEEK(&H6B5)*64 |
| --- | --- |
| 20 | IF K<32 THENK1=8:K2=K-K1 |
| 30 | IF K=32 OR K=64 THENK1=32:K2=K-K1 |
| 40 | IF K=40THENPOKE&H8000,10:POKE0,0:IFPEEK&HA000=10THENK1=32:K2=8 ELSEK1=8: K2=32 |
| 50 | PRINT"Main";K1;TAB(8);"Kbyte Option";K2;"Kbyte";:PRINT:A=64:D=1:A1=0 |
| 70 | GOSUB1000:A=A+A:D=(D+D)MOD256:IF A<K*1024 THEN70 |
| 80 | FORI=1 TO9:A=K*1024-A1-1:IF I=5 OR I=9 THEND=&H80 ELSE D=&H20 |
| 100 | GOSUB1010:IF A1=0 THENA1=1 ELSEA1=A1*2 |
| 110 | NEXT |
| 120 | A=K*1024-A1-1:GOSUB1000:A1=A1*2:IF A1<K*1024 THEN120 |
| 130 | PRINT" OK RAM";:GOSUB10000:PRINT:DEFSEG=&H4000 |
| 160 | ROM=(PEEK(4)+1+PEEK(5)*256)*2 |
| 170 | IF PEEK(0)<>&H43 THENPRINT"NG ROM 1";:GOTO1030 |
| 180 | IF ROM<>1024 THEN200 |
| 190 | D1=PEEK(&H5955):D2=PEEK&HA6AA:D3=PEEK&HFFFF:DEFSEG=&H5000 |
| 191 | D4=PEEK0:D5=PEEK&H5955:D6=PEEK&HA6AA |
| 192 | IF PEEK&HFFFF<>&H20 THENPRINT"NG ROM 2";:GOTO1030 |
| 193 | PRINT"OK 512K*2 ROM?";:PRINT |
| 194 | PRINTRIGHT$(HEX$(D1),2);" ";RIGHT$(HEX$(D2),2);" ";RIGHT$(HEX$(D3),2);" "; RIGHT$(HEX$(D4),2);" ";RIGHT$(HEX$(D5),2);" ";RIGHT$(HEX$(D6),2);"";:GOTO210 |
| 200 | IF PEEK(ROM*128-1)<>&H20 THENPRINT"NG ROM 1";:GOTO1030 |
| 201 | D1=PEEK&H5955:IF ROM=256 THEND2=PEEK&H26AA ELSE D2=PEEK&HA6AA |
| 202 | PRINT"OK";STR$(ROM);"K ROM?";:PRINT |
| 203 | PRINTRIGHT$(HEX$(D1),2);" ";RIGHT$(HEX$(D2),2);" "; |
| 210 | GOSUB10000 |
| 220 | CLS:FOR I=0 TO 63:PRINTCHR$(&HFF);:NEXT:LOCATE 0.0 |
| 230 | DEFCHR$(&HFF)="FFFFFFFFFF":GOSUB10000:DEFCHR$(&HFF)="0000000000": GOSUB10000 |
| 250 | DEFCHR$(&HFF)="AA55AA55AA":GOSUB10000:DEFCHR$(&HFF)="55AA55AA55":TRON: GOSUB10000:TROFF |
| 280 | CLS:PRINT"Printer CHECK";:LPRINT:GOSUB10000 |
| 290 | LPRINT"Main";K1;"Kbyte Option";K2;"Kbyte" |
| 320 | FOR I=32 TO 255:IF I=&H7F THENI=&H80 |
| 330 | LPRINTCHR$(I);:NEXT:LPRINT |
| 350 | PRINT"RS 232C CHECK";:PRINT:GOSUB10000 |
| 360 | OPEN "COM0:5,E,8,2,C,D,C,B,N" AS #1:PRINT#1,123:INPUT #1,D |
| 390 | CLOSE:IF D<> 123 THENPRINT"NG RS 232C";:GOTO1030 |
| 410 | PRINT"OK RS 232C";:GOSUB10000:PRINT |
| 420 | PRINT"MT CHECK <REC>";:PRINT:GOSUB10000 |
| 430 | OPEN "CAS0:TEST" FOR OUTPUT AS #1:PRINT#1,"ABC":CLOSE |
| 460 | PRINT"MT <REW> and <PLAY>";:PRINT:GOSUB10000 |
| 470 | OPEN "CAS0:TEST" FOR INPUT AS #1:INPUT #1,A$:CLOSE |
| 490 | IF A$<>"ABC" THENPRINT"NG MT";:PRINT:GOTO1030 |
| 510 | PRINT"OK MT";:PRINT |
| 520 | END |
| 1000 | POKE A,D |
| 1010 | IF PEEKA=D THEN RETURN |
| 1020 | IF A>=K1 THENPRINT"NG RAM 2"; ELSE PRINT "NG RAM 1"; |
| 1030 | BEEP:GOTO1030 |
| 10000 | BEEP:IF INPUT$(1,@)=CHR$(0) THEN10000ELSE RETURN |

Library: 0800 		Printer Program (character loading)

| 10 | DATA1E,28,A8,28,1E,0,2,7C,80,0,8,4,FE,80,80,0,20,40,80,0,C6,AA,92,82,C6,72,8E,80,8E,72,AA,54,AA,54,AA,FE,FE,FE,FE,FE,1C,22 ,22,1C,22,7E,88,A4,A4,58,20,1E,20,40,40,14,2A,2A,2A,0,3C,52,5 2,3C,0,3E,8,8,30,8,1C,22,22,5C,80,18,26,3C,64,18,F8,88,F8,0,0 |
| --- | --- |
| 20 | DATA48,F8,8,0,0,B8,A8,E8,0,0,A8,A8,F8,0,0,E0,20,F8,0,0,E8,A8,B8,0,0,F8,A8,B8,0,0,C0,80,F 8,0,0,F8,A8,F8,0,0,E8,A8,F8,0,0,20,70,20,0,0,20,20,20,0,0,78,20,40,38,0,48,50,28,48,0,20, 20,0,0,F8,10,10,54,10,10 |
| 30 | DATAA,4A,2A,1A,A,A,1A,2A,4A,A,28,68,38,2C,28,20,40,FE,40,20,1 0,38,54,10,10,8,4,FE,4,8,10,10,54,38,10,22,3C,20,3E,22,18,3A,7 E,3A,18,38,7C,3E,7C,38,18,3C,7E,3C,18,18,7A,7E,7A,18,7C,44,44 ,44,7C,38,44,44,44,38,8,18,28,18,8,40,20,10,8,4,44,28,10,28,44 |
| 40 | DATAFE,90,F0,92,FE,24,DC,54,7E,54,FE,A8,A8,AA,FE,FE,92,92,92,FE,50,50,7E,90,10,84,F 8,A0,A2,BC,12,7E,92,82,42,38,44,FE,44,28,22,22,FA,22,22,88,88,BE,88,88,3E,22,3E,0,0, 0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0 |
| 50 | CLS:PRINT"Down load .....";:a$=CHR$(27):b$=CHR$(0):LPRINTa$;":";b$;b$;b$;a$; "%"; CHR$(1);b$;:x=128:GOSUB60:x=224:GOSUB60:CLS:END |
| 60 | LPRINTa$;"&";b$;CHR$(x);CHR$(x+31);:FORi=x TOx+31:LPRINTCHR$(139);: FORj=1TO5:READc$:LPRINTCHR$(VAL("&H"+c$));b$;:NEXT:LPRINTb$;:NEXT: RETURN |

Library: 0900 		PB100 to FX-850P Converter

| 10 | MODE8:DEFSEG=0:GOTO110 |
| --- | --- |
| 20 | GOSUB20 |
| 30 | OPENp$ FORINPUTAS#1:IFASC(INPUT$(1,1))<>2 THENCLOSE:GOTO30 ELSERETURN |
| 50 | IFg$="" THENRETURN ELSEGOSUB70:FORk=1 TOLEN(g$):k$=MID$(g$,k,1): MODE32(k$): NEXT:g$="":FORl=0 Tok-2:g$=g$+CHR$(PEEK(l+768)):NEXT: RETURN |
| 70 | POKE768,0:POKE769,0:POKE1608,0:POKE1609,3:RETURN |
| 90 | n=PEEK(e):e=e-1:RETURN |
| 110 | n$="":a=2:a$="":j$="":i=0:p$="":m=512:f$="":c$="":o=0:g=0:e=0:d=0:e$="" |
| 120 | p=0:b$="":k=0:g$="":c=0:w=0:u=0:v=0:s=0:o$="":i$="":q=0:h$="":r=0:m$="":k$="":h=0:f=0: t=0:l=0:b=0:x=0:y=0:n=0:d$="" |
| 200 | IFPEEK(m)=32 THENm=m+1:GOTO200 |
| 210 | IFPEEK(m)=4 ANDPEEK(m+1)=180 THENf$="VF":GOSUB2190:GOSUB2120: GOSUB2220: GOTO410 |
| 220 | IFPEEK(m)<>4 ORPEEK(m+1)<>179 THENa |
| 230 | n=PEEK(m+a):IFn=32 THENa=a+1:GOTO230 |
| 240 | IFn=0 ORn=34 ORn=48 ORn=49 THENf$="PF":GOSUB2190:GOSUB2120: GOSUB2220: GOTO410 |
| 250 | IFn=7 ANDPEEK(m+a+1)=187THENf$="AF":a=a+2:GOSUB2190:GOSUB2120: GOSUB2220:GOTO410 |
| 260 | IFn=35 THENf$="MF":a=a+1:GOSUB2190:GOSUB2120 ELSEa |
| 270 | n=PEEK(m+a) |
| 280 | IFn=0 THENg=0:GOTO410 |
| 290 | IFn=32 THENa=a+1:GOTO270 |
| 300 | IFn=44 THENa=a+1:GOSUB2250:IFn=77 THENg=1:GOSUB2220 ELSEa ELSEa |
| 410 | a=1709:o=(PEEK(a+1))*256+(PEEK(a))-1:f=o:e=o:h=o-FRE(1)+1 |
| 430 | FORd=1867 TO1874:e$=e$+CHR$(PEEKd):NEXT |
| 460 | b$="":a$="":OPENp$ FORINPUTAS#1:p=ASC(INPUT$(1,1)):IFp<144 THENCLOSE: GOTO460 |
| 470 | b$=INPUT$(10,1):CLOSE |
| 490 | IFp MOD16=15 THENa$="":GOTO510 |
| 500 | FORk=1 TO8:j$=MID$(b$,k,1):IFj$<>CHR$(7) THENa$=a$+j$:NEXT |
| 510 | i=ASC(MID$(b$,10)):c=VAL("&h"+MID$(HEX$(p),3,1))-8 |
| 520 | IFc=<2 THENn$="MF":GOTO560 |
| 530 | IFc=6 THENn$="VF":GOTO560 |
| 540 | IFc=3ORc=7 THENn$="AF":GOTO560 |
| 550 | n$="PF" |
| 560 | g$=a$:GOSUB50:b$=g$:PRINTb$;TAB(9);n$;:PRINT |
| 570 | IFf$<>n$ THEN460 |
| 580 | IFc$<>"" ANDc$<>b$ THEN460 |
| 600 | ONc GOTO810,,,,810,810,810 |
| 710 | GOSUB30 |
| 720 | n=ASC(INPUT$(1,1)):IFn=255 ORn=241 THEN730 ELSEd$=d$+CHR$(n):GOTO720 |
| 730 | CLOSE:g$=d$:GOSUB50:d$=g$ |
| 735 | IFLEN(d$)>8 THENREAD$ |
| 740 | IFLEN(d$)<8 THENd$=d$+CHR$(0):GOTO740 |
| 750 | IFe$=d$ THEN810 |
| 760 | PASSd$ |
| 810 | GOSUB30:MODE35(:CLOSE:f=PEEK(1609)*256+PEEK(1608)-1 |
| 1010 | IFf$="VF" THEN10010 ELSEPRINT"Converting..."; |
| 1020 | IFf$="MF" THEN6010 |
| 1030 | IFf$="PF" THEN1140 |
| 1110 | w=48:o$=HEX$(PEEK(1715)+PEEK(1716)*256-15):POKE1616,VAL("&h"+RIGHT$( o$,2)): POKE1617,VAL("&h"+LEFT$(o$,2)) |
| 1130 | FORk=48 TO57:k$=CHR$(k):MODE30(k$):NEXT:GOTO1160 |
| 1140 | w=PEEK(PEEK(1611)*256+PEEK(1610)+8):k$=CHR$(w):MODE30(k$) |
| 1160 | q=0:x=0:y=0:l=0:GOSUB70:GOSUB2040:u=n:GOSUB90:v=n:o$=RIGHT$(HEX$(v),2)+ RIGHT$(HEX$(u),2):o$=MID$(STR$(VAL(o$)),2):s=PEEK(1609)*256+PEEK(1608) |
| 1170 | FORk=1 TOLEN(o$):POKE768+k-1,ASC(MID$(o$,k,1)):s=s+1:NEXT:POKE768+k-1,0: i$=HEX$(s):POKE1609,VAL(LEFT$(i$,2)):POKE1608,VAL(RIGHT$(i$,2)) |
| 1180 | GOSUB90 |
| 1182 | IFn=255 THEN1240 |
| 1184 | IFn=254 THENq=0:x=0:y=0:l=0:GOTO1225 |
| 1186 | IFn=7 THENy=y+1:IFy>=2 THENy=0:l=1:GOTO1225 |
| 1188 | IFy=1 THEN1225 |
| 1190 | IFn=167 THENx=1:GOTO1225 |
| 1192 | IFn*x*l=93 THENn=94:GOTO1223 |
| 1194 | IFn=165 ORn=151 THENq=SGN(n-151):GOTO1223 |
| 1196 | IFn*q=94 THENn=151:q=0 |
| 1223 | l=0 |
| 1225 | IFPEEK(1608)>=247 THEN5000 |
| 1230 | h$=CHR$(n):MODE32(h$):GOTO1180 |
| 1240 | GOSUB2080:MODE31(m$):GOTO1160 |
| 2030 | IFPEEK(m+a)=44 ANDPEEK(m+a+1)=77 THENg=1:RETURN |
| 2040 | e=o |
| 2050 | GOSUB90:IFn=224 THENw=w+1:GOTO2050 |
| 2060 | IFn=240 THEN END ELSERETURN |
| 2080 | m$=CHR$(w) |
| 2090 | r=ef:t=o-r+1:MODE33(f+1,t,r):f=t-1:IF(FRE(1))-r<0 THEN20 |
| 2100 | RETURN |
| 2120 | n=PEEK(m+a):IFn=44 ANDf$="MF" THENRETURN |
| 2130 | IFn=0 THEN RETURN |
| 2140 | IFn=32 THENa=a+1:GOTO2120 |
| 2150 | IFn<>34 THENa ELSEa=a+1 |
| 2160 | n=PEEK(m+a):IFn=34 THENa=a+1:c$=LEFT$(c$,8):RETURN |
| 2170 | IFn=0 THENc$=LEFT$(c$,8):RETURN ELSEc$=c$+CHR$(n):a=a+1:GOTO2160 |
| 2190 | n=PEEK(m+a):IFn=48 ORn=49 THENp$="PMT"+CHR$(n)+":":a=a+1:RETURN |
| 2195 | IFn=32 THENa=a+1:GOTO2190 |
| 2200 | p$="PMT0:":RETURN |
| 2220 | n=PEEK(m+a):IFn=0 THENRETURN |
| 2230 | IFn=32 THENa=a+1:GOTO2220 ELSEa |
| 2250 | n=PEEK(m+a):a=a+1:IFn=32 THEN2250 ELSERETURN |
| 5000 | j$=j$+"ST error":GOTO5000 |
| 6010 | IFg=1THEN6040 |
| 6020 | RESTORE# |
| 6030 | RESTORE#"",1,6060:WRITE#:GOTO6030 |
| 6040 | RESTORE#CHR$(26),0,6060:READ$ |
| 6060 | GOSUB70:e=o |
| 6070 | GOSUB90:IFn=240 THEN END |
| 6075 | IFPEEK(1608)>=247 THEN5000 |
| 6080 | IFn<>255 THENh$=CHR$(n):MODE32(h$):GOTO6070 |
| 6090 | GOSUB2090:k=768:s=256*PEEK(1609)+PEEK(1608):l$="" |
| 6100 | IFk=s THENWRITE#l$:GOTO6060 ELSEl$=l$+CHR$(PEEK(k)):k=k+1:GOTO6100 |
| 10010 | e=o:j=0:a=0:IFi\16>0 THEN10100 |
| 10020 | IFa=0 THENPRINT"GET ";(ef-1)/8;"data";:PRINT:PRINT "Converting..."; |
| 10030 | IFe<f+5 THENEND |
| 10040 | GOSUB70:n=PEEK(e-7):IFn>10 THEN10070 |
| 10050 | FORk=0 TO7:GOSUB90:POKE768+k,n:NEXT:MODE34("A("+STR$(j)+")"+CHR$(0)): j=j+1: GOTO10030 |
| 10070 | a=n\16 |
| 10080 | FORk=0 TO7:GOSUB90:a=a-1:IFa>=0 THENMODE32(CHR$(n)) |
| 10085 | NEXT |
| 10090 | MODE34("A$("+STR$(j)+")"+CHR$(0)):j=j+1:GOTO10030 |
| 10100 | GOSUB90:b=n:a=30:IF(i\16)=14 THENa=62 |
| 10110 | PRINT"GET $,";(efa-2)/8;"data";:PRINT:PRINT"Converting...";:GOSUB70 |
| 10120 | FORk=0 TOa:GOSUB90:b=b-1:IFb>=0 THENMODE32(CHR$(n)) |
| 10130 | NEXT:MODE34("$"+CHR$(0)):GOTO10030 |

LLAMADAS  DE SISTEMA: MODOS ESPECIALES

The special modes shown here are not documented in the manual. It is possible, from within BASIC, to execute system commands such as deleting entire programs or inserting new lines of code.

These commands have the format MODE 3x, where x is the special command.

These modes cannot actually be executed directly from RAM, although they can be executed from the function library ROM. Therefore, a routine must be implemented to handle the basic procedures:

- Jump to ROM.

- Execute the MODE 3x command

- return to our program

It is absolutely necessary not to alter or change the number of some lines of the program that will use the special mode.

Commands that affect the editing of other programs (inserting lines, deleting, etc.) must be executed from a subordinate program or within the same program (used for subsequent lines). For example, we can insert lines into program 3 from program 2.

This is because, starting from a lower-level program, memory can be reassigned (by moving memory blocks) easily without altering the execution of the program in use.

The special modes are MODE30 ( ), MODE31( ), MODE32 ( ), MODE33 ( ).

MODE30( )

Delete the specified program as if we were using the NEW command in BASIC.

Syntax:

MODE30( k$ )

k$ : Parameter of type string (text string).

Enter a value between “0” and “9” in k$.

Note: Using a variable other than ak$ is not acceptable. (You can use another variable, but it must be assigned to the variable k$ afterward. E.g., P$="0":k$=P$)

k$ must be a character, not an integer. (e.g., 0 is incorrect; you must use “0”.)

Character saving will be used.

10 k$=”5”:GOSUB 20:END:REM delete program 5

20 GOSUB 21

21 DEFSEG=0:a=PEEK1685+256*PEEK1686:FORb=0TO7:POKEa+b,

VAL(MID$(“20424700510004116024004”,b*3+1,3:NEXT:FORb=0

TO1:Ifb;RETURNELSEa=PEEK1682+256*PEEK1683:POKEa,128:

POKEa+1,24:POKEa+2,4:RETURN:NEXT

Note: You must delete from a program preceding the one you wish to delete. Programs following the deleted program will shift forward to fill the space left.

MODE31( )

Enter the ASCIIZ format line at address 768 in the specified program.

Syntax:

MODE31( m$ )

m$ : Parameter of type string (text string).

Enter a value between “0” and “9” in m$.

The line to be inserted is placed in the VALF function's buffer (at address 768). The line will be in ASCIIZ format, that is, a text string terminated with the value 0.

The lines to be implemented in the program we run will be:

Initialize the variable m$ with a value between “0” and “9” (e.g., m$=”5”)

1140 GOSUB1150

1150 DEFSEG=0:p=PEEK1685+256*PEEK1686:POKEp+5,215:POKEp+6

:POKEp+7,4:POKE1608,0:POKE1609,3:RETURN

1160 RETURN

Put the line "enter" at memory address 768

MODE32( )

Similar to MODE30.

Syntax:

MODE32( k$ )

k$ : Parameter of type string (text string).

Enter a value between “0” and “9” in k$.

Note: Using a variable other than ak$ is not acceptable. (You can use another variable, but it must be assigned to the variable k$ afterward. E.g., P$="0":k$=P$)

k$ must be a character, not an integer. (e.g., 0 is incorrect; you must use “0”.)

Character saving will be used.

10 k$=”5”:GOSUB 20:END:REM delete program 5

20 GOSUB 21

21 DEFSEG=0:a=PEEK1685+256*PEEK1686:FORb=0TO7:POKEa+b,

VAL(MID$(“ 204247005050000200117004 ”,b*3+1,3:NEXT:FORb=0

TO1:Ifb;RETURNELSEa=PEEK1682+256*PEEK1683:POKEa, 212 :

POKEa+1, 17 :POKEa+2,4:RETURN:NEXT

MODE33( )

Copying a block of bytes from one part of memory to another.

Syntax:

MODE33(Param1,Param2,Param3)

Param1: represents the source memory address.

Param2: represents the destination memory address.

Param3: represents the source memory address plus an offset.

(Param3-Param1)=number of bytes to copy.

Because it is a hidden function, it must be called in a somewhat special way. The input parameters must be fixed variables.

Param1= f+1

Param2= t

Param3= r

All variables used are of integer type.

It is mandatory to pass the parameters with the indicated variable names.

MODE33(f+1,t,r)

Graphical Representation

Example: Copy from memory area 0:1000h, 17 bytes, to memory area 0:1

Param1 				Param3=Param1+17

MEMORIA

0:1000h

0:1

Param2

Detailed description

This function cannot be accessed directly from BASIC or CAL mode. Therefore, it must be accessed by transferring control to a program from the library.

It will be necessary to prepare some specific lines in the program that wants to use this system call.

This system call is made by the calculator in some programs of the library (not described in the owner's manual), specifically in program 0900 and more precisely in line 2090.

This line behaves in this program as a subroutine called by the GOSUB command.

2090 r=ef:t=o-r+1:MODE33(f+1,t,r):f=t-1 :IF(FRE(1))-r<0 THEN 20

2100 RETURN Subroutine return

Due to the existing parameters before the MODE33() system call, it is necessary to reconfigure these to adapt them to our purposes.

For this we will have to put a line number 20 in our BASIC program with the desired commands.

The following lines may have different numbering than those shown in the next example, but their content must be identical.

20 REM PUT THE DESIRED COMMANDS HERE

25 ON ERROR GOTO 20

30 GOSUB XX :RESUME ZZ 	Line following the XX

XX POKE 1759.92 :POKE 1760.27 :POKE 1761.4 : f=[P]-1 :

e=([desp]-1)+[P] :o=[d]+(ef)-1 :JCSOFT (put any

expression that causes an error.)

ZZ ...........

Being :

[P] the beginning of the source chain,

[desp] the number of bytes to shift from [P] and

[d] the destination memory area where we want to place the block of bytes.

XX and ZZ represent any line numbers (with the condition that ZZ>XX).

NOTES: The variables in brackets [P], [desp], [d] can be replaced with any variables you wish. The other variables must be the same as in the previous example (f, e, o).

The POKE statements on line XX are responsible for placing the memory address of line 2090 of the library program 0900 in the pointer of ON ERROR GOTO. This address is 4:7004, which is expressed as 92+256*27 in memory bank number 4 (segment).

Note: A practical application of MODE31 and MODE33 can be seen in the chapter " Passing a Library to the P9 Program".

ESTRUCTURA INTERNA DE PROGRAMAS Y MEMO

BASIC programs are located after the variables area. Program lines are structured as follows:

Long(Line Length) 		: 1 byte

Num(Line number) 		: Word (2 bytes)

Car(Space character): ASCII byte(32)

BASIC(BASIC Instructions): [Lon] bytes

Zero (Character 0) 		: 1 byte

The program is equivalent to a structure made up of “n” lines plus the ASCII character(0).

To find out where one of the nine available programs begins, the following formula will be used:

P= PEEK 1715+256* PEEK 1716-15*( PROGRAM +1)

START= PEEK P+256* PEEK (P+1

END= PEEK (P+3)+256* PEEK (P+4

Where PROGRAM is the program number (0-9).

START is the memory area where the program begins.

FIN is the program area where the program ends.

A representation of a program could be the following.

|  | Lon | Num | Car | BASIC (coded) | Zero |
| --- | --- | --- | --- | --- | --- |
| START | x | ab | 32 | ……………………. | 0 |
| START+x+1 | and | CD | 32 | ……………………. | 0 |
| START+x+y+2 | z | ef | 32 | ……………………. | 0 |
| … | … | … | … | … | … |
| END | 0 |

Within each line of the BASIC code, there are special codes to identify or separate the instructions. These are as follows:

| Character | Description |
| --- | --- |
| 1 | A colon ( :) to separate instructions |
| 2 | Start of comment (REM) or (') |
| 3 | It represents jump instructions (GOTO, GOSUB, ON, THEN, ELSE). It is accompanied by a 2-byte number or a variable. |
| 4,5,6,7 | TOKENS (encoded BASIC instructions) |

BASIC instruction structure

The calculator stores each BASIC structure in memory in a specific way.

For certain instructions, there is a reserved data stack in memory. Stacks exist for the GOSUB and FOR …NEXT instructions.

GOSUB Table

It is a stack structure. It starts at address &HB80, incrementing by 8 bytes (see the Pointer Table chapter to find the head and tail of the stack).

Dirección+segmento del puntero del programa llamador.

Nº de línea del GOSUB

Dirección+segmento del separador tras el GOSUB ( : , 0 )

Memoria

Cada cuadro es un byte

FOR table…

The stack for this instruction is arranged in 26-byte increments (see the Pointers Table chapter to find the head and tail of the stack).

FOR Control variable =Initial Value TO Final Value [ STEP increment]: … : NEXT

Dirección+segmento separador tras el NEXT

Nº línea del FOR

Dirección+segmento de la variable de control

Memoria

Dirección+segmento separador tras el FOR

Step

Valor Final

Database (MEMO)

The database consists of ASCII text strings plus carriage return and line feed.

The entry point in memory is located using pointers (see the Pointers Table chapter to find the beginning and end of MEMO ).

The structure is as follows:

Start of MEMO 		“Record 1”, 13, 10

“Record 2”, 13, 10

………………………

“Record n”, 13, 10

26

MEMO 		x ending

Note: The last byte of MEMO is ASCII code 26.

CODIFICACIÓN DE LOS COMANDOS BASIC

BASIC commands are stored in the calculator's memory in coded form, thus occupying less physical space in RAM.

The size occupied by each instruction is two bytes.

The first byte indicates the type of instruction (general BASIC programming, mathematical operations, text string operations and keyboard input, and finally logical, loop, and comparison operators).

The second byte is the instruction identifier.

Based on the first byte, we will have the following values depending on the type:

| Worth | Guy |
| --- | --- |
| 04 | General BASIC Programming |
| 05 | Mathematical operations |
| 06 | Text string operations and keyboard input |
| 07 | Logical, loop, and comparison operators |

Alphabetical list of instructions:

A

| Instruction | Code | ASCII character |
| --- | --- | --- |
| ABS | 5,123 | _{ |
| ACS | 5,111 | _either |
| ALL | 7,187 | _» |
| And | 7,196 | _TO |
| ANGLE | 4,110 | _n |
| ACE | 7,188 | _ ¼ |
| ASC | 5,148 | _” |
| ASN | 5,110 | _n |
| ATN | 5,112 | _p |

B

| Instruction | Code | ASCII character |
| --- | --- | --- |
| BEEP | 4,112 | _p |

C

| Instruction | Code | ASCII character |
| --- | --- | --- |
| CALC$ | 6,173 | _ |
| CALCJMP | 4,159 | _AND |
| CHR$ | 6,160 | _ |
| CLEAR | 4,106 | _j |
| CLOSE | 4,114 | _r |
| CLS | 4,113 | _q |
| COS | 5,108 | _l |
| CSR | 7,184 | _¸ |
| CUR | 5.99 | _c |

D

| Instruction | Code | ASCII character |
| --- | --- | --- |
| DATA | 4,128 | _€ |
| DEF | 4,118 | _v |
| DEFM | 4,119 | _w |
| DEFSEG | 4,120 | _x |
| SDR | 5.56 | _8 |
| DIM | 4,124 | _ ¦ |
| DMS$ | 6,151 | _— |

E

| Instruction | Code | ASCII character |
| --- | --- | --- |
| EDIT | 4,111 | _either |
| ELSE | *7.72 | _H |
| END | 4,135 | _‡ |
| EOF | 5,138 | _¸ |
| ERASE | 4,133 | _… |
| ERL | 5.79 | _EITHER |
| ERR | 5.80 | _P |
| MISTAKE | 4,134 | _† |
| EXP | 5,121 | _and |

F

| Instruction | Code | ASCII character |
| --- | --- | --- |
| FACT | 5,103 | _g |
| FIX | 5,126 | _~ |
| FOR | 4,129 | _ |
| TAILS | 5,127 | _ |
| FRE | 5,141 | _ |

G

| Instruction | Code | ASCII character |
| --- | --- | --- |
| GOSUB | 4.74 | _J |
| GOTO | 4.73 | _YO |

H

| Instruction | Code | ASCII character |
| --- | --- | --- |
| HEX$ | 6,163 | _£ |
| HYP | 5,172 | _¬ |
| HYPACS | 5,117 | _or |
| HYPASN | 5,116 | _t |
| HYPATN | 5,118 | _v |
| HYPCOS | 5,114 | _r |
| HYPSIN | 5,113 | _q |
| HYPTAN | 5,115 | _s |

I

| Instruction | Code | ASCII character |
| --- | --- | --- |
| IF | 4,141 | _ |
| INKEY$ | 6,168 | _¨ |
| INPUT | 6,155 | _› |
| INT | 5,125 | _} |

K

| Instruction | Code | ASCII character |
| --- | --- | --- |
| KEY | 6,169 | © |

L

| Instruction | Code | ASCII character |
| --- | --- | --- |
| LEFT$ | 6,158 | _ž |
| LEN | 5,149 | _• |
| LET | 4,143 | _ |
| LIST | 4.87 | _W |
| LIST | 4.88 | _X |
| LN | 5,119 | _w |
| LOAD | 4.89 | _AND |
| LOG | 5,120 | _x |
| LPRINT | 4,164 | _¤ |

M

| Instruction | Code | ASCII character |
| --- | --- | --- |
| MID | 6,154 | _š |
| MID$ | 6,156 | _œ |
| MOD | 7,199 | _Ç |
| MODE | 4,176 | _ ° |

N

| Instruction | Code | ASCII character |
| --- | --- | --- |
| NCR | 5,171 | _« |
| NEW | 4,107 | _k |
| NEXT | 4,130 | _‚ |
| NOT | 7,195 | _TO |
| NPR | 5,170 | _ª |

O

| Instruction | Code | ASCII character |
| --- | --- | --- |
| ON | 4,154 | _š |
| OPEN | 4,151 | _— |
| OR | 7,194 | _TO |
| OUT | 4,153 | _™ |

P

| Instruction | Code | ASCII character |
| --- | --- | --- |
| PASS | 4.83 | _S |
| PBGET | 4,180 | _´ |
| PBLOAD | 4,179 | _³ |
| PEEK | 5,134 | _† |
| PI | 5.96 | _` |
| POKE | 4.99 | _c |
| POL | 5,168 | _¨ |
| PRINT | 4,163 | _£ |
| PUT | 4,165 | _¥ |

R

| Instruction | Code | ASCII character |
| --- | --- | --- |
| RAN# | 5,147 | _“ |
| READ | 4,168 | _¨ |
| REC | 5,167 | _§ |
| REM | 4,169 | © |
| RESTORE | 4.77 | _M |
| SUMMARY | 4.76 | _L |
| RETURN | 4.75 | K |
| RIGHT$ | 6,157 | _ |
| RND | 5,145 | _ ' |
| ROUND | 5,144 | _ |
| RUN | 4,109 | _m |

S

| Instruction | Code | ASCII character |
| --- | --- | --- |
| SAVE | 4,108 | _l |
| SET | 4,172 | _k |
| SGN | 5,124 | _¦ |
| WITHOUT | 5,107 | _k |
| SQR | 5,122 | _z |
| STEP | 7,192 | _TO |
| STOP | 4,174 | _® |
| STR$ | 6,161 | _! |

T

| Instruction | Code | ASCII character |
| --- | --- | --- |
| TAB | 7,182 | _¶ |
| SO | 5,109 | _m |
| THEN | 7.71 | _G |
| TO | 7,193 | _TO |
| TROFF | 4.95 | _- |
| TRON | 4.93 | _] |

V

| Instruction | Code | ASCII character |
| --- | --- | --- |
| VAC | 4,122 | _z |
| VAL | 5,150 | _– |
| VALF | 5,146 | _' |
| VAR | 4,178 | _² |
| VERIFY | 4.96 | _` |

W

| Instruction | Code | ASCII character |
| --- | --- | --- |
| WRITE# | 4.78 | _N |

X

| Instruction | Code | ASCII character |
| --- | --- | --- |
| XOR | 7,198 | _Æ |

| Table of codes (TOKENS): Numerical order |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | First Byte: |  |  |  |
| 2nd Byte | 4 | 5 | 6 | 7 |
| 71 |  |  |  | THEN |
| 72 |  |  |  | ELSE |
| 73 | GOTO |  |  |  |
| 74 | GOSUB |  |  |  |
| 75 | RETURN |  |  |  |
| 76 | SUMMARY |  |  |  |
| 77 | RESTORE |  |  |  |
| 78 | WRITE# |  |  |  |
| 79 |  | ERL |  |  |
| 80 |  | ERR |  |  |
| 81 |  |  |  |  |
| 82 |  |  |  |  |
| 83 | PASS |  |  |  |
| 84 |  |  |  |  |
| 85 |  |  |  |  |
| 86 |  |  |  |  |
| 87 | LIST |  |  |  |
| 88 | LIST |  |  |  |
| 89 | LOAD |  |  |  |
| 90 |  |  |  |  |
| 91 |  |  |  |  |
| 92 |  |  |  |  |
| 93 | TRON |  |  |  |
| 94 |  |  |  |  |
| 95 | TROFF |  |  |  |
| 96 | VERIFY | PI |  |  |
| 97 |  |  |  |  |
| 98 |  |  |  |  |
| 99 | POKE | CUR |  |  |
| 100 |  |  |  |  |
| 101 |  |  |  |  |
| 102 |  |  |  |  |
| 103 |  | FACT |  |  |
| 104 |  |  |  |  |
| 105 |  |  |  |  |
| 106 | CLEAR |  |  |  |
| 107 | NEW | WITHOUT |  |  |
| 108 | SAVE | COS |  |  |
| 109 | RUN | SO |  |  |
| 110 | ANGLE | ASN |  |  |
| 111 | EDIT | ACS |  |  |
| 112 | BEEP | ATN |  |  |
| 113 | CLS | HYPSIN |  |  |
| 114 | CLOSE | HYPCOS |  |  |
| 115 |  | HYPTAN |  |  |
| 116 |  | HYPASN |  |  |
| 117 |  | HYPACS |  |  |
| 118 | DEF | HYPATN |  |  |
| 119 | DEFM | LN |  |  |
| 120 | DEFSEG | LOG |  |  |
| 121 |  | EXP |  |  |
| 122 | VAC | SQR |  |  |
| 123 |  | ABS |  |  |
| 124 | DIM | SGN |  |  |
| 125 |  | INT |  |  |
| 126 |  | FIX |  |  |
| 127 |  | TAILS |  |  |
| 128 | DATA |  |  |  |
| 129 | FOR |  |  |  |
| 130 | NEXT |  |  |  |
| 131 |  |  |  |  |
| 132 |  |  |  |  |
| 133 | ERASE |  |  |  |
| 134 | MISTAKE | PEEK |  |  |
| 135 | END |  |  |  |
| 136 |  |  |  |  |
| 137 |  |  |  |  |
| 138 |  | EOF |  |  |
| 139 |  |  |  |  |
| 140 |  |  |  |  |
| 141 | IF | FRE |  |  |
| 142 |  |  |  |  |
| 143 | LET |  |  |  |
| 144 |  | ROUND |  |  |
| 145 | LOCATE | RND |  |  |
| 146 |  | VALF |  |  |
| 147 |  | RAN# |  |  |
| 148 |  | ASC |  |  |
| 149 |  | LEN |  |  |
| 150 |  | VAL |  |  |
| 151 | OPEN |  | DMS$ |  |
| 152 |  |  |  |  |
| 153 | OUT |  |  |  |
| 154 | ON |  | MID |  |
| 155 |  |  | INPUT |  |
| 156 |  | SDR | MID$ |  |
| 157 |  |  | RIGHT$ |  |
| 158 |  |  | LEFT$ |  |
| 159 | CALCJMP |  |  |  |
| 160 |  |  | CHR$ |  |
| 161 |  |  | STR$ |  |
| 162 |  |  |  |  |
| 163 | PRINT |  | HEX$ |  |
| 164 | LPRINT |  |  |  |
| 165 | PUT |  |  |  |
| 166 |  |  |  |  |
| 167 |  | REC |  |  |
| 168 | READ | POL | INKEY$ |  |
| 169 | REM |  | KEY |  |
| 170 |  | NPR |  |  |
| 171 |  | NCR |  |  |
| 172 | SET | HYP |  |  |
| 173 |  |  | CALC$ |  |
| 174 | STOP |  |  |  |
| 175 |  |  |  |  |
| 176 | MODE |  |  |  |
| 177 |  |  |  |  |
| 178 | VAR |  |  |  |
| 179 | PBLOAD |  |  |  |
| 180 | PBGET |  |  |  |
| 181 |  |  |  |  |
| 182 |  |  |  | TAB |
| 183 |  |  |  |  |
| 184 |  |  |  | CSR |
| 185 |  |  |  |  |
| 186 |  |  |  |  |
| 187 |  |  |  | ALL |
| 188 |  |  |  | ACE |
| 189 |  |  |  |  |
| 190 |  |  |  |  |
| 191 |  |  |  | STEP |
| 192 |  |  |  | TO |
| 193 |  |  |  |  |
| 194 |  |  |  | NOT |
| 195 |  |  |  |  |
| 196 |  |  |  | And |
| 197 |  |  |  | OR |
| 198 |  |  |  | XOR |
| 199 |  |  |  | MOD |

COMANDOS NUEVOS

This chapter will cover several BASIC commands or instructions that are not documented in the user manual.

It's a shame they weren't included in the manual, as they are very useful.

- Instruction: INPUT@n;Variable

@ -> Wildcard character

n -> number belonging to the range (1<= n <=250)

Variable -> belonging to the range

a~z; A~Z;a$~z$;A$~Z$

Utility: Enter a maximum number n of characters.

The question mark does not appear when requesting the variable.

Example.- 		10 PRINT “Age?”;:INPUT@3;age

With this instruction, there's no need to check that more than three characters are entered. The instruction handles this automatically, saving us from additional programming.

- Instruction: INPUT$( n, @)

n -> number belonging to the range (1<= n <=250)

@ -> Wildcard character.

Use: For selecting options in programs. Eliminates a screen refresh cycle. For using program access keys, as it allows you to enter a maximum number of characters (n) without them being displayed.

Example.- 		5 PRINT”(1) Enter data”TAB(0”(2)Calculate”;

10 op=VAL(INPUT$(1,@)):ONop GOTO100,200:

GOTO 10

100 REM Enter data

200 REM Calculate

The instruction can also be used to cause a pause in the program:

10 PRINT “Press any key to continue”;

20 $=INPUT$(1,@)

30 REM continue the program

- Instruction: CALC$

Utility: To store a formula in a string variable, which saves its contents in the formula storage area (Function Memory), from a BASIC program.

Example.- 		10 CALC$=”AREA=BASE*HEIGHT”

Pressing OUT (Function Memory key) will display the formula.

This variable will normally be accompanied by the CALCJMP function

- Instruction: CALCJMP

Utility: Performs the function of the CALC (Function Memory) key by assigning values to the variables in the formula.

Example.- 		10 CALC$=”AREA=BASE*HEIGHT”

20 CALCJMP

Note: This function should only be used from MODE 0 (CALC Mode). Using it from MODE 1 (BASIC Mode) will generate an invalid function call error (FC error).

- Instruction: DEFCHR$(n)

n -> number, range 252 <=n<=255

Utility: Variable that defines the last 4 characters of the calculator's ASCII code.

For further clarification, see the chapter "Changing ASCII Characters".

Example.- 		10 DEFCHR$(252)=”FFFFFFFFFF”

20 PRINT CHR$(252);

CÓDIGOS DE TECLAS ESPECIALES

Keyboard codes for special keys :

The key code can be obtained with the following statement:

T=ASC(KEY)

| Key | Hexadecimal code |
| --- | --- |
| ENG | 242 |
| MEMO | 251 |
| MODE | 240 |
| CALC | 250 |
| LIB | 252 |
| MENU | 218 |
| S (SHIFT) | 14 |
| IN | 244 |
| OUT | 245 |
| SHIFT+EXE | 253 |
| E (Exponential) | 69 |
| ANS | 246 |

OPTIMIZACIÓN DE CARACTERES

When installing programs, it is possible to reduce the memory used.

Blank spaces between commands are used as wasted bytes.

This space saving can be beneficial for users who do not have the RP-33 memory expansion.

At the same time, saving bytes translates into saving editing time, since we can write almost all the commands and instructions together.

This will result in a loss of readability. Perhaps this programming method should be used by more experienced programmers.

Several examples of byte saving will be shown:

Removing extra parentheses, concatenation symbols, whitespace between instructions, semicolons, double instructions, etc... are several ways to reduce bytes:

Remove parentheses before ( : , THEN , EOL )

Remove quotation marks “ before ( EOL )

Delete; after PRINT “…”

Example 1:

No savings: 		10 PRINT CHR$ (9);"Write your name"+ TAB (0);

With savings: 	10 PRINTCHR$ (9”Write your name” TAB (0;

Example 2:

No savings: 		1000 IF T>=9 THEN R$=”A”

With savings: 	1000 IF T à 9;R$=”A”

Example 3:

No savings: 		200 ON ERROR GOTO 100

With savings: 	200 ONERRORGOTO 100

Example 4:

Without savings: 		50 PRINT Z$;G$;X$;A$;B$;C$;

With savings: 	50 PRINT Z$G$X$A$B$C$;

Example 5:

When there is a conditional statement of the type:

IF … THEN GOTO 100 can be reduced THEN GOTO by GOTO.

Without savings: 		10 IF X>200 THEN GOTO 200

With savings: 	10 IF X>200 GOTO 200

Example 6:

For complex structures, here is the following example.

No savings: 	50 IF B=0 OR G$=”” THEN GOTO 30 ELSE IF X MOD 2+B MOD 2=0 THEN X=X/2

With savings: 	50 F B=0 OR G$=”” GOTO 30 ELSEIF X MOD 2+B MOD 2

=0;X=X/2

There are several operators that can be substituted:

| >= | to |
| --- | --- |
| <= | to |
| <> | to |
| PI | ç |

In the FOR I=A TO C structure, we cannot reduce after the initial variable if it is a letter.

Without saving: 			FOR I=1 TO 10

FOR H=A TO B

With savings: 		FOR I=1 TO 10

FOR H=A TO B ( Note the space between A and TO )

In summary: There are several ways to save bytes in BASIC mode. This saving method is recommended for calculators with limited memory and for more experienced programmers.

CAMBIAR CARACTERES ASCII

The calculator reserves the last four characters of its ASCII code for its use. Using the command DEFCHR$(n) (See chapter New Commands ) you can redefine the characters.

A character displayed on the screen is a 5x7 dot grid, as shown in the figure.

To change a character, you have to separate the character into columns.

The first column is equivalent to a hexadecimal value, the four rows above form half a byte and the three below, plus a virtual row, the other half byte, and so on for the other columns.

Nothing clarifies everything better than an example:

We are going to change the character 252 to the following sign (the symbol of a capacitor):

Separating the columns we will have:

For the active pixel the value is “1”, zero for the off pixel.

23*0=0

22*0=0

21*0=0

20*1=1

23*0=0

22*0=0

21*0=0

20*0=0

&H1

&H0

&H10

Fila virtual

23*0=0

22*1=4

21*1=2

20*1=1

23*1=8

22*1=4

21*0=0

20*0=0

&H7

&HC

&H7C

23*0=0

22*0=0

21*0=0

20*0=0

23*0=0

22*0=0

21*0=0

20*0=0

&H0

&H0

&H00

The other columns are the same as the first two, since the drawing is symmetrical.

Combining all the bytes would give us:

10 DEFCHR$(252)=”107C007C10”

In this way, we have now created character 252. To view it, simply use:

20 PRINTCHR$(252;

This is a program that will help edit characters.

| 8000 | DATA Character Creator (DEFCHR$) |
| --- | --- |
| 8005 | REM redefines the last 4 characters |
| 8010 | CLS:A$="0000000000":X=0:M=128:LOCATE1,1:PRINTCHR$(255; |
| 8020 | C=VALF("&H"+MID$(A$,X*2+1,2 |
| 8030 | B$=LEFT$(A$,X*2)+MID$(HEX$(C),3)+MID$(A$,X*2+3:PRINTCHR$(11CHR$(5B$; |
| 8040 | C$=LEFT$(A$,X*2)+MID$(HEX$(CXORM),3)+MID$(A$,X*2+3:F=0:LOCATE3,1:PRINTMID$("íê",2+(0=(C ANDM,1; |
| 8050 | F=1-F:IFF;DEFCHR$(255)=C$ELSEDEFCHR$(255)=B$ |
| 8060 | T=ASC(KEY:IFT=0GOTO8050ELSEIFT=30ANDM<128;M=M*2ELSEIFT=31ANDM2 ;M=M\2ELSEIFT=29ANDX>0;A$=B$:X=X-1:GOTO8020ELSEIFT=28ANDX<4; A$=B$:X=X+1:GOTO8020 |
| 8065 | IFT>27ANDT<32GOTO8030 |
| 8070 | IFT=13;C=C XORM |
| 8075 | IFT=244GOTO8090 |
| 8076 | IFT=245GOTO8100 |
| 8077 | IFT>64ANDT<91;LOCATE0,1:PRINTCHR$(T; |
| 8080 | GOTO8030 |
| 8090 | CALC$=B$:PRINTCHR$(11CHR$(6"æCALC";:$=INPUT$(1,@:GOTO8030 |
| 8100 | ONERRORGOTO8105:DEFCHR$(252)=CALC$:$="Continue, Delete or CALC (S,B,C)?" :F=1:GOTO8110 |
| 8105 | $="Follow or Delete (S,B) ?":F=0:RESUME8110 |
| 8110 | PRINTCHR$(11CHR$(5$; |
| 8120 | $=INPUT$(1:IF$="S"GOTO8030ELSEIF$="B"GOTO8000ELSEIF$="C"ANDF=1;A$=CALC$:GOTO8020ELSE8120 |

In summary: With a little imagination, you can create characters that suit your needs with relative ease.

PASO DE UNA LIBRERÍA AL PROGRAMA P9

Library programs cannot be listed using normal editing procedures.

One solution to this problem could be to copy a library program into a calculator program.

For ease of programming, we will use BASIC program P9. Using an earlier program would create difficulties when moving subsequent programs in memory. Since this is the last program on the calculator, we only need to focus on this program, not the preceding ones.

There are several ways to transfer a program from the library to the P9. One is manual, and the other is automatic. These programs could, of course, be improved, and they can serve as a starting point for other, more optimized programs.

A manual example would be inserting lines containing data into program P9 until the total size in bytes of the library is reached, and then transferring the information byte by byte to P9. Our program performs the calculation of the data to be inserted.

To avoid having to keep track of the data entered, we will use the Function Memory (Formula Storage Function).

This way we will have to enter “x” lines with the Function Memory data into P9. It is advisable to empty P9 completely, although it is not necessary.

The normal execution program would be: (the number of lines can be whatever you want).

| 4000 | REM Transfers library program to P9 |
| --- | --- |
| 4001 | CLS |
| 4010 | DEFSEG=0:P=PEEK1712+256*PEEK1713:I=PEEKP+256*PEEK(P+1:F=PEEK(P+3)+256*PEEK(P+4 |
| 4015 | CLS:PRINT"Library number ?"TAB(0;:INPUT@5;R$ |
| 4020 | CLS:DEFSEG=5*4096:FORD=63413TO2^16STEP15:S$=" |
| 4030 | FORN=D TOD-1+LEN(R$:S$=S$+CHR$(PEEKN:NEXT |
| 4040 | CLS:PRINTD-7;S$;:IFR$<>S$;NEXT:END |
| 4050 | CLS:D=D-7:A=PEEKD+256*PEEK(D+1:B=PEEK(D+3)+256* PEEK(D+4:IFB-A>FI;CALC$="DATA":FORN=1TO200:CALC$= CALC$+"J":NEXT:PRINT"Must be entered in P9"(BA-F+I)\ 204+1TAB(0"times the OUT line":END |
| 4055 | PRINT"Passing"BA"bytes";TAB(0; |
| 4060 | C=4096*PEEK(D+2:FORN=0TOB-A:DEFSEG=C:B=PEEK(A+N: DEFSEG=0:POKEI+N,B:PRINTCHR$(5N;:NEXT |
| 4070 | BEEP:CLS:END |

Explanation:

In line 4010 we obtain the start and end of P9, by “looking” in the program pointer table (see chapter Pointer Table ).

Line 4015 is the introduction line for the library we want to pass.

The following steps up to 4040 are for checking the existence of the library. Look in the library pointer table, memory segment 5, and check the library number (see the Library Location chapter). If it exists, continue; if it does not, terminate the program.

On line 4050, fill the Function Memory with the command “DATA JJJJJJJJJJJJ…….” (J is an arbitrary value). This tells the user to enter the Function Memory value “x” times in P9.

Once this is done, we will switch to program P9, and in BASIC we will edit the “x” lines as follows:

We will enter EDIT mode and enter a line number, then press the OUT key and then EXE.

It is advisable to include consecutive line numbers for greater clarity (e.g., line numbers 1, 2, 3, 4…x)

Once this process is complete, we return to our program and run it again. If the previous process was performed correctly, the program will proceed to the final lines, transferring the program from the library to P9 byte by byte.

These processes are quite slow due to the speed of the calculator's microprocessor and user interaction.

To reduce user interaction time (editing P9) there is the following program that works automatically.

It is based on the previous program, but performing the editing tasks using system calls (thus avoiding the hassle of editing).

| 10 | CCAL$=CALC$:PRI=1:GOTO 4000 |
| --- | --- |
| 999 | REM Insert a line in P9 (System call MODE31) |
| 1000 | m$="9" |
| 1140 | GOSUB 1150 |
| 1150 | DEFSEG=0:PE=PEEK1685+256*PEEK1686:POKEPE+5,215: |
|  | POKEPE+6,26:POKEPE+7,4:POKE1608,0:POKE1609,3:RETURN |
| 1160 | RETURN |
| 2999 | REM Pass CALC$ to Buffer VALF (system call MODE33) |
| 3000 | a=1024:b=768:c=LEN(CALC$):GOSUB 3005 |
| 3005 | GOSUB 3020:RESUME3010 |
| 3010 | RETURN |
| 3020 | DEFSEG=0:ONERRORGOTO 3020:POKE1759.92:POKE1760.27: POKE1761,4:f=a-1:e=c+f:o=b+c-1:jcsoft |
| 4000 | CLS:REM Passes library program to P9 |
| 4010 | DEFSEG=0:P=PEEK1712+256*PEEK1713:I=PEEKP+256*PEEK(P+1:F=PEEK(P+3)+256*PEEK(P+4 |
| 4015 | IFPRI;CLS:PRINT"Library number ?"TAB(0;:INPUT@5;R$:PRI=0 |
| 4020 | CLS:DEFSEG=5*4096:FORD=63413TO2^16STEP15:S$=" |
| 4030 | FORN=D TOD-1+LEN(R$:S$=S$+CHR$(PEEKN:NEXT |
| 4040 | CLS:PRINTD-7;S$;:IFR$<>S$;NEXT:END |
| 4050 | CLS:D=D-7:A=PEEKD+256*PEEK(D+1:B=PEEK(D+3)+256*PEEK (D+4:IFB-A>FI;CALC$="DATA":FORN=1TO200:CALC$=CALC$+ "J":NEXT:PRINT"Entering in P9"(BA-F+I)\204+1TAB(0" times the OUT line";:in=(BA-F+I)\204+1:GOTO 5000 |
| 4055 | PRINT"Passing"BA"bytes";TAB(0;:ONERRORGOTO 4060 |
| 4060 | C=4096*PEEK(D+2:FORN=0TOB-A:DEFSEG=C:B=PEEK(A+N: DEFSEG=0:POKEI+N,B:PRINTCHR$(5N;:NEXT |
| 4070 | BEEP:CLS:CALC$=CCAL$:END |
| 5000 | FOR R=1 TO in:AA$=CALC$:CALC$=STR$(R)+" "+CALC$: GOSUB 3000:GOSUB1000:CALC$=AA$:NEXTR |
| 5010 | GOTO 4000 |

Explanation:

Everything stated for the previous program is valid except for the editing.

The automatic process involves filling the CALC$ variable in Function Memory with the line number and moving it to memory address 768 (where the VALF function buffer is located) using the MODE33 system call (see the System Calls chapter). Once this is done, the line is loaded into P9 using the MODE31 system call. Finally, the library is transferred to P9 byte by byte.

ACCESO A LIBRERÍAS DESDE BASIC

To access the program libraries from calculator mode (MODE 0), type the library number and then press the LIB key.

From within a program we can also access the libraries, through a series of instructions.

Instruction: GOTO”LIB0:Num”,

…THEN”LIB0:Num”,

GOSUB”LIB0:Num”

Num -> Library number (see chapter Library Location )

Description: 	Runs the Num library from BASIC

Example 1: 		5 REM executes the library menu

10 GOTO”LIB0:K0630

Example 2: 		5 REM executes array library

100 IFa THEN”LIB0:5100

Example 3: 		5 REM jumps to a statistics library

10 GOSUB”LIB0:S6410

Using a non-existent library number will generate an undocumented error:

| Error code | Error message | Meaning | Correction |
| --- | --- | --- | --- |
| 10 | NF Error (NF Error) (Error Not Found) | The specified library was not found (from BASIC mode) | Enter an existing library number |

ELIMINAR CONTRASEÑA

Sometimes using a password carries the risk of it being forgotten.

The password is stored in RAM as an 8-byte ASCIIZ string, i.e., an 8-byte text string plus the zero byte.

The method for removing the password is very simple.

The first thing to do is locate the memory address where it is stored.

The second step is to fill the first byte of that address with the character zero.

For the first step in the Pointer Table chapter , we can find the address where the password is stored. This address is 1867.

Simply use the command POKE1867,0 and the password will be removed. It's that simple.

OCULTAR EL BANCO DE DATOS(MEMO)

This section is used to hide the data bank (MEMO), making it appear empty.

There are a few things to keep in mind when hiding MEMO. Once hidden, it's essential not to write to the database again, as this can destabilize the system.

The procedure is similar to password removal (see chapter "Password Removal "). It involves changing the first byte of MEMO to zero (or to another byte less than 32). 	It is recommended to write a space character in the first byte of MEMO before changing it to zero.

A possible program would be the following:

| 10 | CLS:DEFSEG=0 |
| --- | --- |
| 20 | P=PEEK1703+256*PEEK1704 |
| 30 | IFPEEKP=0;PRINT”Hidden or empty memory”TAB(0”Do you want to retrieve it? (Y/N)?”;:INPUT@2;$:IF$=”S”OR$=”s”;POKEP,32:END |
| 40 | PRINT”Do you want to hide the MEMO(Y/N)?”;:INPUT@2;$:IF$=”S”OR$=”s” ;POKEP,0:END |

LONGITUD DE LOS PROGRAMAS

No function has been implemented to keep track of the memory used by programs.

However, this unofficial guide has provided a solution for that.

To find out the length of the programs, the program pointers can be used (See chapter Pointer Table ).

[xxxx] Lo que hay en la dirección xxxx (varía según la cantidad de RAM

Each program structure consists of 15 bytes of information. These range from P9 to P0, followed by the information from the Data Bank (MEMO).

[1712]

P9

P8

P7

P6

P5

P4

P3

P2

P1

P0

MEMO

[1715]

[1709]

15 bytes

“P”

“5”

INICIO

BL

FINAL

BL

80

32

32

32

32

32

32

An example of a program to find out the length of programs could be the following:

Note: We are not reading the BL information, as the segment is in RAM (DEFSEG=0). If we wanted to do the same with the libraries, we would have to use DEFSEG=4*4096 or DEFSEG=5*4096.

| 10 | CLEAR:DEFSEG=0:GOSUB100:P=PEEK1712+256*PEEK1713+DESP |
| --- | --- |
| 20 | I=PEEKP+256*PEEK(P+1 |
| 30 | F=PEEK(P+3)+256*PEEK(P+4 |
| 40 | IFR;R$="MEMO"ELSER$="P"+P$ |
| 50 | CLS:PRINT"Length: Physical="FI;TAB(8": Logical="FI-1" bytes";:LOCATE2,1:PRINTR$: GOTO10 |
| 100 | CLS:PRINTCHR$(9"Program length"TAB(0"(M)EMO /N. PROGRAM ?";: INPUT@2;P$ |
| 110 | PP=VAL(P$ |
| 120 | IFP$="M";DESP=-15:R=1:RETURN |
| 130 | IFPP<0ORPP>9GOTO100 |
| 140 | IFASC(P$>57ORASC(P$<48GOTO100 |
| 150 | DESP=15*(15-(PP+6:RETURN |

CONECTOR DE LA AMPLIACIÓN DE RAM (RP-33)

The RP-33 expansion is a hardware component of the calculator. Its topology is not mentioned in the user manual.

Below is a brief description of the connections to the calculator:

1

1 /2

16   conectores

14 conectores

|  | SPI | 1 | VPP | Feeding |
| --- | --- | --- | --- | --- |
| Read/Write | W/R | 2 | VPP | Feeding |
| Chip Select 2 | CS2 | 3 | CS3 | Chip Select 3 |
| Chip Select | CS | 4 | AD12 | Address 12 |
| Address 8 | AD8 | 5 | AD7 | Address 7 |
| Address 9 | AD9 | 6 | AD6 | Address 6 |
| Address 11 | AD11 | 7 | AD5 | Address 5 |
| Output Enable | OE | 8 | AD4 | Address 4 |
| Address 10 | AD10 | 9 | AD3 | Address 3 |
| Chip Select 1 | CS1 | 10 | AD2 | Address 2 |
| Data 8 | ID8 | 11 | AD1 | Address 1 |
| Data 7 | ID7 | 12 | AD0 | Address 1 |
| Data 6 | ID6 | 13 | ID1 | Data 1 |
| Data 5 | ID5 | 14 | ID2 | Data 2 |
| Data 3 | ID3 | 15 | Vss | 0 Volts (Ground) |
| Data 4 | ID4 | 16 | Vss | 0 Volts (Ground) |

CONECTOR DE PERIFÉRICOS

The connection diagram is as follows:

1

1

2

2

3

3

4

4

5

5

6

6

7

7

8

8

9

9

10

10

11

11

12

12

13

13

14

14

15

15

+5 V

Entrada LOAD/RS 232C

Salida SAVE/RS 232C

A cero al abrirse un canal de RS 232 C

BUSY. Impresora preparada para recibir datos

PSTB. Petición de impresora. (Print Strobe)

Control REM Cassette.

=0 cassette en marcha

Calculadora ON/OFF

=0 Calculadora encendida

BUSY. Impresora preparada para recibir datos

IO4

IO3

IO2

IO1

IO5

IO6

IO7

IO8

AD1 bit 1 cursor

OE

bit 0 cursor AD0

bit 2 cursor AD2

RD/WR

GND 0V

2 pasos a cero por cada carácter a impresora

The calculator's peripheral connector is not a standard connector. CASIO has its own data transmission interface and is not keen on selling separate connectors.

It is a 30-pin connector (15 on top - 15 on the bottom, completely parallel).

The signals from this connector are TTL levels from 0 to 5V compared to those of an RS-232 connector.

It serves both for serial communication and for sending data to the printer.

Index

INTRODUCTION

RAM MEMORY ADDRESSING

POINTER TABLE

VARIABLE AND POINTER STORAGE

ROM MEMORY LIST

LOCATION OF THE LIBRARY (BOOKSTORES)

UNDOCUMENTED BOOKSTORES

SYSTEM CALLS: SPECIAL MODES

INTERNAL STRUCTURE OF PROGRAMS AND MEMO

BASIC COMMAND CODING

NEW COMMANDS

SPECIAL KEY CODES

CHARACTER OPTIMIZATION

CHANGE ASCII CHARACTERS

TRANSITION FROM A LIBRARY TO THE P9 PROGRAM

ACCESS TO LIBRARIES FROM BASIC

REMOVE PASSWORD

HIDE THE DATABASE (MEMO)

PROGRAM LENGTH

RAM EXPANSION CONNECTOR (RP-33)

PERIPHERAL CONNECTOR